Internet of Things (IoT)

Theory & Algorithm Reference

Practicals 1 – 4

Name: Prajwal Dudhal  |  Roll No: 13154  |  Batch: A3

Practical 1 ¦ IR Sensor Obstacle Detection using LEDs

| Practical No. | 1 |
| --- | --- |
| Title | IR Sensor Obstacle Detection using LEDs |
| Aim | To understand the connectivity of Raspberry Pi with IR sensor and write an application to detect obstacles and notify the user using LEDs. |

## Theory

- An IR (Infrared) sensor detects objects by emitting infrared light and measuring the reflected signal.

- When an obstacle is present, the IR beam reflects back to the sensor and the output pin goes LOW (0).

- When no obstacle is present, the output pin stays HIGH (1) as no reflection is received.

- The Raspberry Pi reads the sensor output via a GPIO input pin (Pin 36).

- Two LEDs are used: RED LED (Pin 40) lights up when obstacle is detected, GREEN LED (Pin 37) lights up when path is clear.

- GPIO.BCM or GPIO.BOARD mode is used to reference pins by board position or BCM number respectively.

## Algorithm

- Set GPIO mode and configure Pin 36 as INPUT, Pins 40 and 37 as OUTPUT.

- In a loop, read the value of Pin 36 (IR sensor output).

- If value == 0 (obstacle detected): turn ON RED LED (Pin 40), turn OFF GREEN LED (Pin 37), print 'OBJECT DETECTED'.

- If value == 1 (no obstacle): turn OFF RED LED, turn ON GREEN LED (Pin 37), print 'OBJECT NOT DETECTED'.

- Sleep for 0.5 seconds and repeat.

| Input | IR Sensor output: randomly simulated as 0 (detected) or 1 (not detected) |
| --- | --- |
| Output | OBJECT DETECTED... [RED LED ON] / OBJECT NOT DETECTED... [GREEN LED ON] |

Conclusion: The IR obstacle detection application was successfully implemented and simulated. The program correctly reads sensor values and toggles LEDs to notify the user about obstacle presence.

Practical 2 ¦ Temperature Sensor with LED Alerts

| Practical No. | 2 |
| --- | --- |
| Title | Temperature Sensor with LED Alerts |
| Aim | To understand connectivity of Raspberry Pi with a DHT11 temperature/humidity sensor and generate LED alerts when temperature or humidity crosses a threshold. |

## Theory

- The DHT11 is a low-cost digital sensor that measures both temperature and humidity.

- It communicates with the Raspberry Pi over a single-wire protocol using a GPIO data pin.

- The Adafruit_DHT library provides read_retry() to reliably fetch sensor readings.

- Two thresholds are defined: TEMP_THRESHOLD = 28°C and HUMID_THRESHOLD = 65%.

- Two LEDs are used: one for temperature alert (Pin 20) and one for humidity alert (Pin 26).

- GPIO.BCM mode is used; the sensor data pin is connected to GPIO 21.

## Algorithm

- Set GPIO mode BCM; configure Pin 20 and Pin 26 as OUTPUT.

- In a loop, read temperature and humidity using Adafruit_DHT.read_retry(11, 21).

- If humidity > 65: turn ON humidity LED (Pin 26); else turn it OFF.

- If temperature > 28: turn ON temperature LED (Pin 20); else turn it OFF.

- Print current temperature and humidity values.

- Repeat every 1 second.

| Input | Simulated DHT11 readings: Temperature in range 24–35°C, Humidity in range 55–80% |
| --- | --- |
| Output | Humidity: 65.7% ¦ Temperature: 30.3°C → HUMIDITY HIGH [HUMIDITY LED ON] → TEMP HIGH [TEMP LED ON] ⚠ ALERT! |

Conclusion: The temperature and humidity monitoring application was successfully implemented. LED alerts were correctly triggered whenever sensor readings crossed the defined threshold values.

Practical 3 ¦ Camera Interface — Capture Image and Record Video

| Practical No. | 3 |
| --- | --- |
| Title | Camera Interface — Capture Image and Record Video |
| Aim | To understand the connectivity of Raspberry Pi with PiCamera and write an application to capture and store images and videos. |

## Theory

- The Raspberry Pi Camera Module connects via the CSI (Camera Serial Interface) port and is controlled using the picamera Python library.

- camera.capture('image.jpg') captures a still image and saves it to the specified file path.

- camera.start_recording() and camera.stop_recording() are used to record video in H264 format.

- LEDs are used as status indicators: LED on Pin 40 indicates image capture in progress; LED on Pin 38 indicates video recording.

- The time.sleep() function controls the recording duration.

- Files are saved with timestamps to uniquely identify each capture.

## Algorithm

- Initialize GPIO pins 40 and 38 as OUTPUT; initialize PiCamera object.

- Turn ON LED (Pin 40); call camera.capture() to take a photo; save to 'image.jpg'.

- Sleep 1 second; turn OFF LED (Pin 40); print confirmation.

- Turn ON LED (Pin 38); call camera.start_recording() to begin video capture.

- Sleep 3 seconds (recording duration); call camera.stop_recording().

- Turn OFF LED (Pin 38); print confirmation.

| Input | No external input — camera module triggered by program execution |
| --- | --- |
| Output | ✓ Image Captured — saved as: captured_images/image_20260509_165821.jpg ✓ Video Recorded — saved as: captured_videos/video_20260509_165825.h264 |

Conclusion: The PiCamera application was successfully implemented and simulated. Images and videos were captured and saved with correct timestamps, and LED indicators worked as expected during capture and recording.

Practical 4 ¦ IoT Cloud Dashboard — ThingSpeak Publisher/Subscriber

| Practical No. | 4 |
| --- | --- |
| Title | IoT Cloud Dashboard — ThingSpeak Publisher/Subscriber |
| Aim | To create a small dashboard application deployed on cloud (ThingSpeak) where publisher devices can publish sensor data and subscriber applications can monitor it. |

## Theory

- ThingSpeak is an IoT cloud platform that allows devices to send and receive data over the internet using HTTP.

- A publisher device sends sensor data (temperature, humidity) to a ThingSpeak channel via HTTP POST requests.

- Subscribers can read the channel data via HTTP GET requests or the ThingSpeak dashboard.

- Each channel has a unique Write API Key used to authenticate data publishing.

- Data fields (field1, field2...) map to individual sensor values in the channel.

- The application uses Python's http.client to make POST requests to api.thingspeak.com:80/update.

## Algorithm

- Configure ThingSpeak channel key and channel ID.

- In a loop, read temperature and humidity from DHT11 sensor.

- Build URL-encoded parameters: field1=temperature, field2=humidity, key=API_KEY.

- Open HTTP connection to api.thingspeak.com:80 and send POST request to /update.

- Read and print the HTTP response status.

- Sleep 5 seconds (ThingSpeak requires minimum 15 seconds between updates in free tier).

| Input | Simulated sensor: Temperature 24–38°C, Humidity 50–85% ¦ Channel Key: HJEQMRUDGXPTXNPY |
| --- | --- |
| Output | [HTTP POST] Status: 200 OK ¦ Entry ID: 1 ThingSpeak Dashboard shows entry with Temp and Humidity values |

Conclusion: The IoT cloud dashboard application was successfully implemented and simulated. Sensor data was published to ThingSpeak at regular intervals and the dashboard correctly displayed all published records.